*************************

*        IDTHEIDE       *

*************************

IDTHEIDE is an executable file that will identify any IDE drive at

drive address 0 or drive address 1. Syntax for IDTHEIDE is as

follows:

IDTHEIDE < /Dx >

where /Dx is optional and selects drive 0 (/D0) or drive 1 (/D1). If

/Dx is not included, drive 0 is assumed.

Information returned from the drive includes the following:

1. Drive manufacturer and model number.

2. Drive serial number.

3. Firmware revision.

4. Buffer size.

5. Controller type.

6. Buffer type (only one of the following possible):

1. Single ported single sector buffer without

simultaneous transfers to or from the host

and disk.

2. Dual ported multiple sector buffer with

simultaneous transfers to or from the host

and disk.

3. Dual ported multiple sector buffer capable

of simultaneous transfers and read cacheing.

4. Not specified.

7. Number of cylinders.

8. Number of heads.

9. Number of sects/track.

10. Number of bytes/track.

11. Number of bytes/sect.

12. Number of ECC bytes.

13. Multiple command block size.

14. DMA cycle timing mode.

15. PIO cycle timing mode.

16. Doubleword I/O support.

17. Other information (any of the following possible):

- Not a magnetic disk drive.

- Format speed tolerance gap required.

- Track offset option available.

- Data strobe offset option available.

- Motor speed tolerence  > 0.5% & < 1%.

- Disk transfer rate > 10Mb/s.

- Disk transfer rate > 5Mb/s & <= 10Mb/s.

- Disk transfer rate <= 5Mb/s.

- Removable cartridge drive.

- Fixed disk drive.

- Spindle motor control option available.

- Head switch time > 15us.

- Not MFM encoded.

- Soft sectored.

- Hard sectored.

For more details on Identify Drive information, refer to section 9.4

of the ATA (AT Attachment) revision 2.3.

IDTHEIDE version 3.1 changes DRQ and DRSC timeouts to 10 seconds (old

versions were set to 2 seconds).

IDTHEIDE version 3.2 now ignores the index bit when polling for DRSC

and DRQ.

#